Доступ в горячий коридор контейнера

1) Отключить контейнер.

2) Открыть большие торцевые двери контейнера (вентиляционное отделение).

3) Открутить 6-8 болтов крепления по периметру вертикальной вентиляционной решетки, см. фото.

4) Отодвинуть и прислонить вентиляционную решетку вправо вплотную к воздушному фильтру.

Обратная процедура

5) Вернуть вентиляционную решетку на место и закрутить болты крепления. Проверить что решетка установлена ровно.

6) Проверить провода от верхнего и нижнего электропривода решетки

7) Включить контейнер, проверить что вентиляционная решетка работает

8) Закрыть большие торцевые двери контейнера (вентиляционное отделение).

-------

Hot tunnel access

1) Disable container.

2) Open the large end doors of the container (ventilation compartment).

3) Unscrew 6-8 fastening bolts around the perimeter of the vertical ventilation grill, see photo.

4) Move and lean the ventilation grill to the right close to the air filter.

Reverse procedure

5) Return the ventilation grill to its place and tighten the fastening bolts. Check that the grille is installed evenly.

6) Check the wires from the upper and lower electric drive of the grill

7) Switch on the container, check that the ventilation grill is working

8) Close the large end doors of the container (ventilation compartment).