How the alarm system works.

Outside there are four cameras, four radars and two speakers.

Inside the office building there are four cameras and two speakers
Inside each container there are one camera and a door sensor.

Container:

The magnetic door sensor is always on. If you open the container door without deactivating, the alarm will be triggered. To deactivate the door sensor, you must use a small NFC “tag” on a panel in the office building, or an app on your phone. When it`s deactivated, it will stay deactivated until 11PM when it`s activated automatically. You can manually activate it at any time. 
Me and Even is the only ones that are configured in the system at present time.

Outside:

Cameras and radars are activated at 11 PM. If something triggers the alarm, the alarm center will connect to see what triggered the alarm. If there is an intruder, they will try to scare him/them off with enouncing over the speakers that the police are notified. Simultaneously they will contact the police and start calling the person that are number 1 on the contact list.

Office building:

There are no sensors at the doors or windows. There are four cameras that register movement. When there are registered movement, the alarm center will do the same procedure as outside.

Fire detection:

There are sensors installed I each container and office building. If the fire alarm is triggered, the fire department will automatically be notified. The alarm center will also connect to see if there actually is a fire. Persons on the list will also be contacted.

In case of sabotage/error:

If they loose connection with any equipment, the alarm center will connect and make a check. If needed they will trigger the alarm to the police/fire department.

How the alarm system works.

Contact list:

If an alarm is triggered, the following people in numbered order will be alerted by Relisec. The password you will be asked for are: Fjell IT er best)

- Jonny Andersen (Fjell IT AS, Owner)

- Even Andre Solheim (Fjell IT AS, Technician)

- Finn Berntsen (Dataroom AS, Chairman)

(They will try to contact nr. 1, if he doesn`t answer, the will try number 2 and so on.)

Door sensor on containers:

What time the sensor activates, can be changed.

Looking at the outdoor cameras:

Download and install “XProtect Smart Client 64-bit from the link below.
https://www.milestonesys.com/support/resources/download-software/?prod=3&type=11&ver=1499&lang=27

Computer:		82.134.73.26
Authentication: 	Basic authentication	
User Name:		Alvdal
Password:		Xh11122

You will be able to pan, tilt and zoom live and also look at recordings.